Underwater Acoustic Synthetic Dataset

Physics-Based Vessel Noise Signatures

Technical Delivery Document

For Self-Supervised Learning Applications

in Underwater Acoustics

Version 5.0.0

January 2026

# Abstract

This document describes the completed Stage -1 of the SKANN-SSL project: a synthetic underwater acoustic waveform generation system. The V5 system produces physics-motivated vessel noise signatures combined with Knudsen-model sea noise, generating a full-factorial structured dataset of 12,000 five-second clips covering all combinations of design factors. Key V5 improvements include non-overlapping shaft rate ranges for acoustic distinguishability, exactly three structural resonances per clip, and corrected swell frequencies for realistic cavitation modulation. The output is ready for ingestion by Stage 0 (Preprocessing & Data Standardisation).

# Executive Summary

Stage -1 of the SKANN-SSL project has been successfully completed with the V5 dataset. The deliverables include:

Sea Noise Generator: Piecewise parametric Knudsen model for 4 sea states

Ship Noise Generator: Hybrid spectral/physical model with 6 acoustic components

Cavitation Model: Physical burst model at 200 kHz with blade-gated timing

Full-Factorial Dataset: 12,000 structured clips covering all design factor combinations

Preprocessing Pipeline: Tensor generation ready for Stage 0/1 ingestion

Comprehensive Metadata: 27-column CSV manifest with all parameters and measurements

## Key Metrics

| Metric | Value |
| --- | --- |
| Total Clips | 12,000 |
| Vessel Clips | 9,600 (4 classes x 2,400) |
| No-Vessel Clips | 2,400 |
| Clip Duration | 5.0 seconds |
| Sampling Rate | 16,000 Hz |
| Samples per Clip | 80,000 |
| Signal-to-Noise Ratio | 6.0 dB |
| Analysis Band | 10 - 8,000 Hz |

# Project Context

## SKANN-SSL Pipeline Overview

The SKANN-SSL system is a hybrid selective kernel self-supervised acoustic representation learning system. Stage -1 provides the synthetic waveforms for system bring-up before real-world datasets are integrated.

Pipeline stages: Stage -1 (Synthetic Data) -> Stage 0 (Preprocessing) -> Stage 1 (SKConv1D) -> Stage 2 (Encoder) -> Stage 3 (SSL/Barlow Twins) -> Stage 4 (Augmentation) -> Stage 5 (Training) -> Stage 6 (Evaluation) -> Stage 7 (Deployment)

## Stage -1 Objectives

Per the project roadmap, Stage -1 objectives were:

1. Generate synthetic vessel-noise waveforms using a Python-based generator

2. Produce physics-inspired acoustic signatures including blade-rate tonals, broadband propulsion noise, and cavitation bursts

3. Implement non-overlapping shaft rate ranges for acoustic distinguishability (V5 critical fix)

4. Save generated signals at 16 kHz sampling rate

5. Organise into a structured dataset with comprehensive metadata

All objectives have been met.

# Technical Implementation

## System Architecture

The generation pipeline consists of two parallel branches that are combined at a controlled SNR:

Branch 1 - Sea Noise Generation: Knudsen CSV data -> Piecewise Parametric Model -> Colored Noise via FFT/IFFT

Branch 2 - Ship Noise Generation: Vessel Parameters -> Tonal Synthesis (shaft, BPF, generator, equipment, resonances) + Broadband Flow Noise + Cavitation Bursts (200 kHz physical model)

Both branches are combined at SNR Mix (6 dB) to produce the final output waveform in Pascals.

## V5 Critical Improvements

### 1. Non-Overlapping Shaft Rate Ranges

Previous versions had overlapping boundaries that made vessel classes acoustically indistinguishable at certain shaft rates. V5 enforces strict separation:

| Vessel Class | Shaft Rate (Hz) | RPM Range | Gap to Next |
| --- | --- | --- | --- |
| tanker | 1.00 - 1.50 | 60 - 90 | --- |
| cargo_ship | 1.50 - 2.50 | 90 - 150 | 0.0006 Hz |
| fishing_vessel | 4.00 - 8.00 | 240 - 480 | 1.50 Hz |
| small_craft | 15.01 - 30.00 | 900 - 1800 | 7.01 Hz |

The large gaps between fishing_vessel and small_craft (7 Hz) and cargo_ship and fishing_vessel (1.5 Hz) ensure acoustic distinguishability.

### 2. Exactly Three Resonances Per Clip

V5 enforces exactly 3 structural resonances per vessel clip:

| Resonance Band | Frequency Range | Physical Source |
| --- | --- | --- |
| Band 1 | 50 - 150 Hz | Hull modes |
| Band 2 | 100 - 300 Hz | Foundation/mounting |
| Band 3 | 200 - 500 Hz | Piping/ductwork |

### 3. Corrected Swell Frequency

V5 uses realistic ocean swell periods for cavitation modulation:

V5 (correct): 0.05 - 0.15 Hz corresponding to 6.7 - 20 second periods

Previous (incorrect): 0.5 Hz corresponding to unrealistic 2-second period

## Sea Noise Model

Sea noise is generated using a piecewise parametric model based on digitised Knudsen curves.

### Knudsen Model

The noise level (NL) in each frequency band follows:

NL(f) = a * log10(f) + b   [dB re 1 uPa^2/Hz]

where a (slope) and b (intercept) are fitted from digitised Knudsen curves for each sea state.

### Frequency Bands

| Band | Frequency Range | Physical Mechanism |
| --- | --- | --- |
| Turbulence | 10 Hz - f_t | Hydrodynamic turbulence (flat plateau) |
| LF | f_t - 200 Hz | Wind/mechanical (rising) |
| MF | 200 - 500 Hz | Transition shoulder |
| HF | 500 - 8000 Hz | Wind-driven decay |

The turbulence-LF transition frequency f_t is approximately 30-40 Hz depending on sea state.

### Sea State Levels

| Sea State | Description | Sea RMS (Pa) | Sea SPL (dB) |
| --- | --- | --- | --- |
| 0 | Calm | 0.0076 | 77.6 |
| 1 | Light air | 0.0151 | 83.6 |
| 3 | Gentle breeze | 0.0479 | 93.6 |
| 6 | Strong breeze | 0.1514 | 103.6 |

## Ship Noise Model

Ship noise is generated using a hybrid approach: spectral synthesis for deterministic components and a physical burst model for cavitation.

### Vessel Classes

| Vessel Class | Shaft Rate (Hz) | Cavitation Peak (Hz) | Broadband Rolloff |
| --- | --- | --- | --- |
| small_craft | 15.0 - 30.0 | 5000 | -3 dB/octave |
| fishing_vessel | 4.0 - 8.0 | 1500 | -4 dB/octave |
| cargo_ship | 1.5 - 2.5 | 600 | -5 dB/octave |
| tanker | 1.0 - 1.5 | 400 | -6 dB/octave |

### Acoustic Components

The ship noise signal comprises six component types:

1. Shaft Rate Harmonics: Fundamental shaft rotation frequency plus harmonics from mechanical imbalance. Amplitude decays at 3 dB per harmonic.

f_shaft = n_shaft / 60   [Hz]

2. Blade Pass Frequency (BPF) Harmonics: The dominant tonal component. Amplitude decays at 4 dB per harmonic.

f_BPF = n_blades x f_shaft   [Hz]

3. Generator Harmonics: For vessels with electrical generators at 50 Hz or 60 Hz. Level is 10 dB below shaft harmonics.

4. Equipment Running Frequency: Shipboard machinery at 25 Hz or 30 Hz with 4-7 harmonics.

5. Structural Resonances: Three narrowband resonances per clip from ship structure.

6. Broadband Flow Noise: Hydrodynamic turbulence with power-law spectrum decay.

### Cavitation Burst Model

Cavitation is modeled as discrete bubble collapse events using a physical approach:

High-Resolution Generation: Bursts are generated at 200 kHz to capture the microsecond-scale collapse physics, then downsampled to 16 kHz with anti-aliasing.

Blade-Gated Timing: Bursts occur synchronised to blade passage with swell modulation at 0.05-0.15 Hz.

Burst Physics: Collapse time (tau) of 50-200 microseconds following Rayleigh collapse dynamics. Envelope is exponential decay with sinusoidal carrier at vessel-specific peak frequency.

# Full-Factorial Dataset Design

## Design Factors

The structured dataset covers all combinations of design factors:

| Factor | Levels | Count |
| --- | --- | --- |
| Sea State | 0, 1, 3, 6 | 4 |
| Vessel Class | small_craft, fishing_vessel, cargo_ship, tanker | 4 |
| Blade Count | 3, 4, 5 | 3 |
| Generator Frequency | 0 Hz (off), 50 Hz | 2 |
| Cavitation Intensity | 0.0, 0.333, 0.667, 1.0 | 4 |
| Repetitions | 25 | 25 |

Total Vessel Clips: 4 x 4 x 3 x 2 x 4 x 25 = 9,600

No-Vessel Clips: 4 sea states x 600 repetitions = 2,400

Grand Total: 12,000 clips

## Class Distribution

| Class | Clips | Percentage |
| --- | --- | --- |
| small_craft | 2,400 | 20.0% |
| fishing_vessel | 2,400 | 20.0% |
| cargo_ship | 2,400 | 20.0% |
| tanker | 2,400 | 20.0% |
| no_vessel | 2,400 | 20.0% |
| Total | 12,000 | 100% |

## Cavitation Statistics

| Category | Clips | Percentage |
| --- | --- | --- |
| With cavitation (intensity > 0) | 7,200 | 75.0% |
| Without cavitation (intensity = 0) | 2,400 | 25.0% |
| Burst count range | 14 - 749 | --- |
| Peak frequency range | 400 - 5000 Hz | --- |

# Output Specification

## Dataset Location

Google Drive: https://drive.google.com/drive/u/1/folders/1E6vhPnkY8x8YzZ3a-k6PnL_G9gnq5gBo

## File Structure

SKANN_SSL_V5_Dataset/

waveforms/ (12,000 .npy files)

tensors/ (12,000 .npy files)

master_dataset_manifest.csv

pairing_manifest.csv

## File Formats

| Type | Format | Shape | Units |
| --- | --- | --- | --- |
| Waveforms | .npy (Float32) | (80000,) | Pascals |
| Tensors | .npy (Float32) | (1, 1, 80000) | Normalised |
| Manifest | .csv | 12,000 rows x 27 columns | Metadata |

## Manifest Schema (27 Columns)

The manifest contains comprehensive metadata for each clip including: clip_id, repeat_index, sea_state, vessel_class, n_blades, generator_freq, cavitation_intensity, shaft_rate, blade_pass_freq, has_cavitation, cavitation_peak_freq, n_cavitation_bursts, equipment_base_freq, resonance_freq_1, resonance_freq_2, resonance_freq_3, sea_rms_pa, ship_rms_pa, combined_rms_pa, scale_factor, sea_spl_db, ship_spl_db, combined_spl_db, snr_db, filename, tensor_path, waveform_path.

# Validation Results

## SNR Verification

The SNR is maintained at exactly 6.0 dB for all vessel clips, confirming correct implementation of the SNR scaling.

## Shaft Rate Verification

| Vessel Class | Expected Range | Actual Min | Actual Max | Status |
| --- | --- | --- | --- | --- |
| tanker | 1.00 - 1.50 Hz | 1.00 Hz | 1.50 Hz | PASS |
| cargo_ship | 1.50 - 2.50 Hz | 1.50 Hz | 2.50 Hz | PASS |
| fishing_vessel | 4.00 - 8.00 Hz | 4.00 Hz | 8.00 Hz | PASS |
| small_craft | 15.00 - 30.00 Hz | 15.01 Hz | 30.00 Hz | PASS |

No overlap detected between vessel classes. Clean separation confirmed.

## Physical Units Verification

Sea State 3 reference levels:

Sea RMS: 0.0479 Pa (93.6 dB re 1 uPa) - matches Knudsen

Ship RMS: 0.0955 Pa (99.6 dB re 1 uPa) - 6 dB above sea

Combined RMS: 0.1068 Pa (100.6 dB re 1 uPa) - correct power addition

# Integration with Stage 0

## Tensor Format Compatibility

The preprocessed tensors are ready for direct ingestion:

import torch

import numpy as np

tensor = np.load('tensors/tensor_000000.npy')

x = torch.from_numpy(tensor)  # Shape: [1, 1, 80000]

## Stage Boundary

Inputs: Knudsen curve data (SS*CSV.txt), Configuration (config.py)

Outputs: Waveforms (clip_XXXXXX.npy), Tensors (tensor_XXXXXX.npy), Manifest (master_dataset_manifest.csv), Pairing manifest (pairing_manifest.csv)

Consumers: Stage 0 (Preprocessing), Stage 3 (SSL training via pairing manifest)

# Known Limitations

1. Doppler not implemented: Doppler-induced frequency shifts were deprioritised. Can be added as post-processing if needed.

2. Single-source model: Each clip contains one vessel. Multi-source scenarios require extension.

3. Stationary statistics: Each clip has fixed parameters. Time-varying scenarios (e.g., CPA approach) not modeled.

4. No propagation effects: Multipath, surface/bottom reflection, and range-dependent attenuation not included.

5. Synthetic only: Real-world datasets (NOAA, MBARI, JAMSTEC, DCLDE) should be integrated for final validation.

# References

Knudsen, V. O., et al. (1948). Underwater Ambient Noise. Journal of Marine Research.

Ross, D. (1976). Mechanics of Underwater Noise. Pergamon Press.

Urick, R. J. (1983). Principles of Underwater Sound. McGraw-Hill.

Li, X., et al. (2019). Selective Kernel Networks. CVPR 2019.

Zbontar, J., et al. (2021). Barlow Twins: Self-Supervised Learning via Redundancy Reduction. ICML 2021.

Document version: 5.0.0

Dataset version: V5.0.0

Date: January 2026